6502 Program Exercise 01

Input of a string of characters

This program is meant to do the same job as the C command gets – read in a string of characters from the keyboard.

This program will be closely related to program example 4, which was the output of a string.  Especially the way that indexed addressing was used, so I would have that handy while you tackle this.

I want you to write a program which will continually check the keyboard buffer to see if anything has been typed.  When a character has been typed, it should store it in an array of initially empty memory locations, as well as displaying it on the output screen.  The program should end when the return key has been pressed.

The main difference is the little wait loop.  The program will spend most of its time going around that loop waiting for you to type something.

Remember that the location $E004 is the ‘input buffer’ as it were.  If a character has been typed in it will be copied to this location.

The algorithm is as follows:

Clear the terminal window

Initialise the index register

Display a prompt character (such as ‘>’ )

wait:	Get char from keyboard buffer

If zero flag set – keyboard buffer empty – jump back to top of wait loop

Is character a carriage return?  ($0D)

If yes, jump to end of string label eos

Display character in terminal window

Store character in memory.

Increment index register

Jump back to wait for next character.

eos:	Load acc with string termination character (Don’t want to store return character)

Place at end of stored string.

End of program

I would suggest using $01 as a string termination character, simply because you will be able to see it when it is stored in the memory.  Normally you would use $00 but as the memory is full of $00 anyway, it is difficult to see if the termination character has been used or not.